Std_types.h file:

1- typedef unsigned long long u64;

= long long → means it’s a 64-bit integer type on most compilers (like GCC/AVR-GCC, ARM-GCC, etc.).
__________________________________________________________________________________

DIO_interface.h file:

1- // Enable internal pull-up resistor for pin

void DIO_voidEnablePullUp(u8 Copy_u8PortID, u8 Copy_u8PinID);

= That way:

- When the switch or external device is disconnected or open, the pin reads HIGH.

- When the switch connects to GND, the pin reads LOW.

__________________________________________________________________________________

GLCD_int.h file:

1- // External declaration of 5x7 font array

extern const u8 font5x7[][5];

#### = extern

- The extern keyword means this variable is defined in another file, not in the current one.

- It tells the compiler not to allocate memory for it here — it will be provided somewhere else.

#### ➤ const

- The const keyword means the array’s contents cannot be modified at runtime (read-only).

- Fonts are fixed lookup tables, so this is logical — they never change.

#### ➤ u8

- This is your project’s alias for unsigned char, meaning each element is 8 bits (1 byte).

#### ➤ font5x7[][5]

- This is a two-dimensional array, where each element is a 5-byte pattern representing a character (in a 5×7 pixel font).

GLCD_prog.c:

1-     // Generate enable pulse

DIO_voidSetPinValue(GLCD_CTRL_PORT, GLCD_EN_PIN, 1);

_delay_us(5);  // Wait for setup time

DIO_voidSetPinValue(GLCD_CTRL_PORT, GLCD_EN_PIN, 0);

// Deselect both chips

DIO_voidSetPinValue(GLCD_CTRL_PORT, GLCD_CS1_PIN, 0);

DIO_voidSetPinValue(GLCD_CTRL_PORT, GLCD_CS2_PIN, 0);

_delay_us(5);  // Wait for hold time

=   Set EN = 1 → start the enable pulse.

  Wait 5 microseconds → allows data/control signals to stabilize (“setup time”).

  Set EN = 0 → the falling edge triggers the GLCD to latch the data (store it into its internal register).

After finishing a read or write operation:

- Both CS1 and CS2 are set to 0 → meaning no chip is selected.

This ensures:

- No accidental data writes happen.

- The data bus remains stable before the next command.

- The GLCD’s internal timing (hold time) is respected.

2- /* Initialize GLCD hardware and controller */

void GLCD_voidInit(void)

{

// Configure data port as output

DIO_voidSetPortDirection(GLCD_DATA_PORT, 0xFF);

// Configure control pins as output

DIO_voidSetPinDirection(GLCD_CTRL_PORT, GLCD_RS_PIN, 1);

DIO_voidSetPinDirection(GLCD_CTRL_PORT, GLCD_RW_PIN, 1);

DIO_voidSetPinDirection(GLCD_CTRL_PORT, GLCD_EN_PIN, 1);

DIO_voidSetPinDirection(GLCD_CTRL_PORT, GLCD_CS1_PIN, 1);

DIO_voidSetPinDirection(GLCD_CTRL_PORT, GLCD_CS2_PIN, 1);

DIO_voidSetPinDirection(GLCD_CTRL_PORT, GLCD_RST_PIN, 1);

// Reset sequence

DIO_voidSetPinValue(GLCD_CTRL_PORT, GLCD_RST_PIN, 0);

_delay_ms(10);  // Hold reset low

DIO_voidSetPinValue(GLCD_CTRL_PORT, GLCD_RST_PIN, 1);

_delay_ms(50);  // Wait for stabilization

// Turn on both display halves

GLCD_voidCommand(0x3F, 1);  // Display on for chip 1

GLCD_voidCommand(0x3F, 2);  // Display on for chip 2

_delay_ms(100);  // Wait for display ready

// Clear display and reset cursor

GLCD_voidClear();

}

=

| Step | Action | Purpose |
| --- | --- | --- |
| 1 | Set data port to output | MCU can send commands/data to GLCD |
| 2 | Set control pins to output | MCU controls GLCD’s interface |
| 3 | Reset pulse | Ensure GLCD starts clean |
| 4 | Turn on both halves | Activate both display controllers |
| 5 | Clear screen | Start from a blank display |

3-     // Calculate local column within chip (0-63)

u8 local_col = (y < 64) ? y : (y - 64);

// Send position commands to appropriate chip

GLCD_voidCommand(0xB8 | x, chip);      // Set page

GLCD_voidCommand(0x40 | local_col, chip); // Set column

=   If the pixel column (y) is on the left side (0–63) → use it directly.

  If it’s on the right side (64–127) → subtract 64 to map it into the chip’s local coordinate.

  0xB8 is the base address command for setting the page (row group) in the GLCD.

  Each “page” = 8 vertical pixels (a byte).

  The lower 3 bits of this command (bits 0–2) select the page number (0–7).

And same for columns

4-     // Calculate font table index

u8 idx = c - 32;

/* Write each of 5 font bytes */

for (u8 i = 0; i < 5; i++)

=   Each character (like 'A', 'B', 'C', etc.) is stored as a 5-byte pattern.

  Each byte corresponds to one vertical column of pixels in a 5×7 font matrix.

- ' ' (ASCII 32) → index 0 in the table

- 'A' (ASCII 65) → index 33 in the table (65 - 32 = 33)

👉 That’s why idx = c - 32; — it aligns the ASCII code with the font array index.

5-     /* Handle page wrap */

if (current_col >= 128)

{

current_col = 0;

current_page++;

if (current_page >= 8) current_page = 0;

}

=

| Action | Result |
| --- | --- |
| You fill all 128 columns on the current page | You reach the screen’s right edge |
| The code resets column to 0 | Cursor moves to the far left |
| The page increments | Cursor moves down one block (8 pixels lower) |
| If you’re already at page 7 | You wrap around to the top again |

6-      char buf[12];

This line declares an array named buf (short for buffer) that can hold 12 characters.

Main.c file:

1- 	// Calculate on-time pixels for PWM

uint8_t high_px = (uint8_t)(period_px * (duty / 100.0));

// Generate next PWM point (1 pixel wide)

uint8_t bit_val = (t < high_px) ? 1 : 0;

t = (t + 1) % period_px;

= This section simulates a digital PWM signal by:

- Computing how many pixels should represent the “ON” time,

- Determining if the current point t is high or low,

- And cycling through the waveform period continuously.

2- 	for (uint8_t x = 0; x < 128; x++) {

uint8_t y_current = buffer[x] ? y_top : y_bottom;  // current waveform level

uint8_t y_prev = (x > 0) ? (buffer[x-1] ? y_top : y_bottom) : y_current; // previous level

= This line means:

- If buffer[x] == 1, waveform is HIGH, so pixel should be drawn near the top (y_top).

- If buffer[x] == 0, waveform is LOW, so pixel should be drawn near the bottom (y_bottom).

Y_prev is used later

3- 			uint8_t page = yy / 8;

uint8_t bit = yy % 8;

uint8_t chip = (x < 64) ? 1 : 2;

uint8_t local_col = (x < 64) ? x : (x - 64);

=

| Variable | Calculation | Result | Meaning |
| --- | --- | --- | --- |
| page | 23 / 8 | 2 | Page 2 |
| bit | 23 % 8 | 7 | Bit 7 within that page |
| chip | 90 < 64 → false → 2 | 2 | Use chip 2 |
| local_col | 90 - 64 | 26 | Column 26 in chip 2 |